- Connect to SU ECS Linux Server

For windows users, open Microsoft Store and search Ubuntu, install a complete Ubuntu terminal environment in minutes with Windows Subsystem for Linux (WSL).

Open Ubuntu, set root user password. Type

ssh -X yourNetID@ecs-linux.syr.edu

Login using your NetID credentials.

Tips: The server is currently only open for SU ECS students using a campus network. If you are trying to connect with other network environment, the connection will be rejected.

- Transferring files for lab

Download FileZilla client (Download FileZilla Client for Windows (64bit x86) (filezilla-project.org))

Host: ecs-linux.syr.edu with your NetID, Port 22

Drop the lab files and unzip them.

- Run Synopsys Formality

After connecting the Linux server and adding the lab files, type $ qsh to move to the VC Formality server. Type $ syn& to bring up a terminal.

In the new terminal, type the command to start the Formality shell environment.

% fm_shell

fm_shell (setup)>

Start the graphical user interface (GUI) as follows:

fm_shell (setup)> start_gui

Then the window shows up: