- Launch the TetraMAX gui. First change the directory to /synopsys_labs/lab06_tetramax/work, then enter the command to start:

tmax -tcl

- Read the library and netlist file

read_netlist ../ref/lib/stdcell_rvt/verilog/saed14nm_rvt.tv

read_netlist ../source/johnson_dft_compiled.v

- Run build model, name the top module johnson

Type the command or click the toolbar

run_build_model johnson

- Design rule checking

run_drc ../source/Johnson_test.spf -test

- Update fault list. Enter the command

add_faults -all

- Set and run ATPG

set_atpg -capture 10

run_atpg -auto

- Summary and data report

report_summaries

analyze_faults -class au

report_patterns -summary

- Write the patterns

write_patterns ../results/atpg_parallel.stil -format stil -parallel -replace

write_patterns ../results/atpg_serial.stil -format stil  -serial -replace

- Load scripts to automatically perform a set of commands to get the result.

BULD-T> source ../scripts/atpg_pre.tcl

- Full gui instruction:

Don’t forget to click run to read the files!

Using Full-seq test, results

Report summaries

How to improve the test coverage to 100%?